BMWG Working Group             				 F. Calabria

Internet-Draft                                           Cisco

Intended status: Informational                    C. Pignataro

Expires: August 23, 2026         		  Blue Fern Consulting

February 2026

Benchmarking Methodology for AI Training Network Fabrics

draft-calabria-bmwg-ai-fabric-training-bench-00

Abstract

This document defines benchmarking terminology, methodologies, and Key Performance Indicators (KPIs) for evaluating Ethernet-based AI training network fabrics.

As large-scale distributed AI/ML training clusters grow to tens of thousands of accelerators (GPUs/XPUs), the backend network fabric becomes the critical bottleneck determining job completion time (JCT), training throughput, and accelerator utilization.

This document establishes vendor-independent, reproducible test procedures for benchmarking fabric-level performance under realistic AI training workloads, covering RDMA/RoCEv2 transport, the Ultra Ethernet Transport (UET) protocol defined by the UEC Specification 1.0, congestion management (PFC, ECN, DCQCN, CBFC), load balancing strategies (ECMP, DLB, packet spraying), collective communication patterns (AllReduce, AlltoAll, AllGather), and scale/soak testing.

The methodology enables apples-to-apples comparison across different switch ASICs, vendor implementations (multiple vendor implementations), NIC transport stacks (RoCEv2 vs. UET), and fabric architectures (2-tier Clos, 3-tier Clos, rail-optimized).

Status of This Memo

This Internet-Draft is submitted in full conformance with the provisions of BCP 78 and BCP 79. Internet-Drafts are working documents of the Internet Engineering Task Force (IETF). Note that other groups may also distribute working documents as Internet-Drafts. The list of current Internet-Drafts is at https://datatracker.ietf.org/drafts/current/.

Internet-Drafts are draft documents valid for a maximum of six months and may be updated, replaced, or obsoleted by other documents at any time. It is inappropriate to use Internet-Drafts as reference material or to cite them other than as "work in progress."

Copyright Notice

Copyright (c) 2026 IETF Trust and the persons identified as the document authors. All rights reserved. This document is subject to BCP 78 and the IETF Trust's Legal Provisions Relating to IETF Documents (https://trustee.ietf.org/license-info) in effect on the date of publication of this document.

Table of Contents

1. Introduction

1.1. Requirements Language

1.2. Scope and Applicability

1.3. Relationship to Existing BMWG Work

2. Terminology and Definitions

3. Test Topology and Architecture

3.1. Reference Fabric Topologies

3.2. Device Under Test (DUT) Identification

3.3. Traffic Generator Requirements

4. KPI Framework and Metrics Taxonomy

4.1. Primary KPIs

4.2. Secondary KPIs

4.3. Fabric Health Indicators

5. Test Category 1: RDMA Transport Benchmarks

5.1. Baseline Throughput

5.2. Latency Characterization

5.3. Back-to-Back Burst Absorption

6. Test Category 1A: UEC Transport Protocol Benchmarks

6.1. UET Throughput by Transport Service

6.2. UET Latency Characterization

6.3. Packet Spray Efficacy Under UET RUD

6.4. UET Congestion Control Benchmarks

6.5. Link Layer Enhancement Benchmarks

6.6. UET Collective Communication Performance

6.7. UET PDC Scalability and Connection Setup Rate

7. Test Category 2: Congestion Management

7.1. ECN Marking Accuracy and Threshold

7.2. PFC Behavior Under Incast

7.3. DCQCN Convergence Time

7.4. PFC Storm and Deadlock Resilience

8. Test Category 3: Load Balancing Efficacy

8.1. ECMP Entropy and Polarization

8.2. Dynamic Load Balancing (Flowlet)

8.3. Packet Spraying

8.4. Jain Fairness Index Measurement

9. Test Category 4: Collective Communication

9.1. AllReduce Benchmark

9.2. AlltoAll Benchmark

9.3. AllGather Benchmark

9.4. NCCL/Collective Library Bus Bandwidth

10. Test Category 5: Job Completion Time (JCT)

10.1. Synthetic JCT Under Controlled Conditions

10.2. MLPerf-Aligned JCT

10.3. Multi-Tenant JCT Interference

11. Test Category 6: Scale and Convergence

11.1. Fabric Scale Limits

11.2. Link Failure Convergence

11.3. Zero-Impact Failover Measurement

12. Test Category 7: Soak and Stability

12.1. 24-Hour Sustained Load

12.2. Resource Leak Detection

13. Reporting Format

14. Security Considerations

15. IANA Considerations

16. References

Appendix A: KPI Summary Tables

Appendix B: ASIC Feature Matrix

Appendix C: Frame Formats for RoCEv2 Testing

Appendix D: UET Frame Format

Authors' Addresses

1. Introduction

The rapid growth of distributed AI/ML training workloads has fundamentally changed the performance requirements for data center network fabrics. Unlike traditional data center traffic characterized by diverse flow sizes and protocols, AI training workloads generate highly synchronized, bandwidth-intensive, east-west traffic patterns dominated by collective communication operations (AllReduce, AlltoAll, AllGather). These workloads impose unique demands: lossless transport (via RoCEv2 over RDMA), ultra-low tail latency, near-perfect load balancing across all fabric paths, and the ability to absorb coordinated micro-bursts from thousands of accelerators simultaneously.

Existing BMWG methodologies, while foundational, do not adequately address the characteristics of AI training fabrics. RFC 2544 [RFC2544] defines benchmarking for general network interconnect devices but does not account for RDMA transport semantics, collective communication patterns, or the unique congestion dynamics of GPU-to-GPU traffic. RFC8238/RFC 8239 [RFC8238][RFC8239] establish data center benchmarking terminology and methodology but predate the AI fabric paradigm and do not address RoCEv2-specific behaviors such as Priority Flow Control (PFC) interactions, DCQCN congestion control convergence, or the impact of load balancing strategies on Job Completion Time (JCT).

The EVPN benchmarking methodology [EVPN-BENCH] provides a structural template for service-oriented benchmarking but is scoped to L2VPN services rather than RDMA fabrics.

This document fills the gap by defining a comprehensive benchmarking methodology specifically designed for AI training network fabrics. The methodology is vendor-independent and ASIC-agnostic, enabling fair comparison across implementations from current-generation switch silicon including deep-buffer VOQ ASICs, multiple silicon vendors and ASICs including deep-buffer VOQ and others. It covers the full stack from raw RDMA transport performance (both legacy RoCEv2 and the emerging UEC Ultra Ethernet Transport protocol) through collective communication efficiency to end-to-end Job Completion Time, providing operators and evaluators with the metrics needed to select, validate, and optimize AI fabric deployments.

1.1. Requirements Language

The key words "MUST", "MUST NOT", "REQUIRED", "SHALL", "SHALL NOT", "SHOULD", "SHOULD NOT", "RECOMMENDED", "MAY", and "OPTIONAL" in this document are to be interpreted as described in BCP 14 [RFC2119] [RFC8174] when, and only when, they appear in all capitals, as shown here.

1.2. Scope and Applicability

This document applies to Ethernet-based AI training backend network fabrics employing RoCEv2 and/or UEC Ultra Ethernet Transport (UET) protocols. The scope includes leaf-spine (2-tier Clos) and leaf-spine-superspine (3-tier Clos) topologies.

InfiniBand fabrics are explicitly out of scope, though many KPIs defined herein may be adapted for IB benchmarking by future documents. The DUT is the network fabric itself (the collection of switches and interconnecting links), not individual accelerators or host NICs, though host-side configuration (including NIC transport stack: RoCEv2 vs. UET, and UEC compliance profile) MUST be documented as it materially affects results.

The methodology is designed for controlled laboratory environments per the BMWG charter; it is NOT intended for production network measurement.

1.3. Relationship to Existing BMWG Work

This document extends and complements the following BMWG and other specifications:

-------------------------------------------------------------

Document**                 **Relationship

-------------------------------------------------------------

RFC 1242 			Base terminology for network benchmarking; terms reused herein

RFC 2544				Base methodology throughput/latency/loss  tests adapted for RDMA

RFC 2889				LAN switching methodology; MAC learning concepts adapted for ARP/ND scale

RFC 8238				Data center terminology; buffer, congestion, and microburst terms extended

RFC 8239				Data center methodology; line-rate and buffer tests adapted for RoCEv2

RFC 9004				Back-to-back frame updates; burst absorption methodology referenced

draft-gaikwad-llm-* The [LLM-BENCH] document series (draft-gaikwad-llm-benchmarking-methodology) is complementary to this document but addresses a different layer of the AI infrastructure stack. [LLM-BENCH] benchmarks the inference serving stack as a whole — characterizing end-to-end system performance with KPIs expressed in application-visible terms such as Time to First Token (TTFT) and output tokens per second (TPS) — treating the network fabric as an opaque component of the System Under Test (SUT). This document benchmarks the network fabric itself, with the DUT boundary drawn at the switches and interconnecting links, and KPIs (BusBW, JCT Ratio, JFI) that isolate the fabric's contribution to collective communication performance under distributed training workloads. The traffic models also differ fundamentally: [LLM-BENCH] generates bursty, point-to-point inference request traffic, whereas this document generates bulk synchronous collective operations (AllReduce, AllGather, AllToAll) characteristic of gradient synchronization in data-parallel training. The two documents MAY be used together to characterize a fabric across its full operational envelope but MUST NOT be combined in a single benchmarking report without explicit section demarcation.

UEC Spec 1.0						UET protocol specification; transport services, congestion control, and link-layer enhancements benchmarked in Section 6

-----------------------------------------------------------

2. Terminology and Definitions

The following terms are defined for use in this document. Where a term overlaps with RFC 1242 or RFC 8238, the definition herein takes precedence in the context of AI fabric benchmarking.

-------------------------------------------------------------	Term**       	  **Definition

---------------- ---------------------------------------------

AI Fabric		The dedicated Ethernet backend network interconnecting accelerators (GPUs/XPUs) for distributed AI training, typically a non-blocking Clos topology running RoCEv2

JCT				Job Completion Time: the wall-clock duration from the start of a training job (or benchmark iteration) until all participating accelerators complete their work, inclusive of all computation and communication phases

DUT Fabric		The complete system under test: all leaf switches, spine switches, superspine switches (if applicable),and interconnecting links forming the AI training fabric

Roofline JCT	The theoretical minimum JCT assuming perfect(zero-contention) network behavior, computed from computation time plus serialization delay at link rate with zero queuing

JCT Ratio		Measured JCT divided by Roofline JCT; a value of 1.0 indicates no network-induced overhead; values >1.0 indicate fabric inefficiency

BusBW				Bus Bandwidth: the effective data throughput achieved	per accelerator during a collective operation, calculated as (data_size * algo_factor) / time, where algo_factor accounts for the collective algorithm (e.g., 2*(n-1)/n for ring AllReduce)

QP					Queue Pair: the fundamental RDMA communication endpoint comprising a Send Queue and Receive Queue; multiple QPs per source-destination pair increase ECMP entropy

Incast Ratio	The ratio of senders to receivers in a communication pattern, e.g., N:1 incast where N sources simultaneously target one destination port

MMR				Max-Mean Ratio: ratio of the flow count on the most loaded link to the average flow count per link; quantifies ECMP imbalance (1.0 is perfect)

PFC 				Pause Event  A Single Priority Flow Control PAUSE frame transmitted on a priority; indicates the upstream device's buffers are approaching capacity for that priority class

ECN Marking	Ratio	Percentage of packets marked with Congestion Ratio	Experienced (CE) in the IP ECN field over measurement interval

Collective Operation		A coordinated communication pattern executed across Operation all accelerators in a training group: AllReduce (gradient synchronization), AlltoAll (expert routing in MoE), AllGather (parameter distribution)

DCQCN				Data Center Quantized Congestion Notification: the combination of ECN and PFC used with RoCEv2 for end-to-end congestion control

Packet Spray	A load balancing strategy that distributes individual packets of a single flow across all available ECMP paths; maximizes link utilization but may cause packet reordering

DLB/Flowlet		Dynamic Load Balancing using flowlet detection: reroutes traffic when a flow has been idle longer than a flowlet gap threshold, reducing reordering risk vs. packet spray

Zero-Impact		Sub-microsecond automatic path convergence upon link Failover or switch failure, resulting in no measurable impact to JCT

UET			Ultra Ethernet Transport: the connectionless RDMA transport protocol defined by the Ultra Ethernet Consortium (UEC) Specification 1.0, designed as a next-generation replacement for RoCEv2 in AI/HPC fabrics

PDC			Packet Delivery Context: the ephemeral, lightweight transport endpoint in UET, analogous to but distinct from an RDMA Queue Pair (QP); PDCs are connectionless and require no explicit setup handshake before data transmission

ROD			Reliable Ordered Delivery: a UET transport service providing reliable, in-order packet delivery, semantically equivalent to RoCEv2 RC mode; suitable or legacy RDMA applications requiring strict ordering

RUD			Reliable Unordered Delivery: a UET transport service optimized for AI workloads that provides reliable delivery without maintaining packet ordering; enables native packet spraying without reorder buffer overhead at the receiver NIC

RUDI			Reliable Unordered Delivery for Idempotent operations: a UET transport service optimized for operations that are safe to execute more than once (e.g., RDMA Writes), allowing simplified retransmission logic

UUD			Unreliable Unordered Delivery: a UET transport service for best-effort, unordered traffic; lowest overhead, suitable for telemetry or speculative operations

LLR			Link Layer Retry: an optional UEC link-layer enhancement providing fast per-hop error recovery (sub-microsecond) at the Ethernet link layer, reducing the load on transport-layer retransmission

Packet Trimming	  An optional UEC link-layer enhancement where congested switches transmit only the packet header (trimmed packet) instead of dropping the full packet, enabling the receiver to detect loss faster and trigger selective retransmission with reduced bandwidth waste

CBFC			 Credit-Based Flow Control: an optional UEC link-layer mechanism for buffer management using credit-based admission, an alternative to PFC that avoids head-of-line blocking and PFC storm propagation

UEC Profile			A defined subset of UET features targeting a specific use case: AI Base (core AI training), AI Full (AI Base + deferred send, exact-match tagging, extended atomics), or HPC (latency-optimized for traditional HPC)

Entropy Value	A per-packet field in UET used to distribute packets across ECMP paths; provides explicit spray entropy independent of the 5-tuple, improving multipath utilization for AI traffic

--------------------------------------------------------------

3. Test Topology and Architecture

3.1. Reference Fabric Topologies

Three reference topologies are defined. All tests MUST specify which

topology is used. Results obtained under different topologies are NOT

directly comparable without normalization.

3.1.1. Topology A: 2-Tier Clos (Leaf-Spine)

+--------+  +--------+  +--------+  +--------+

| Spine1 |  | Spine2 |  | Spine3 |  | SpineN |

+---++---+  +---++---+  +---++---+  +---++---+

||          ||          ||          ||

||    Full Mesh Interconnect         ||

||    (ECMP / DLB / Spray)          ||

||          ||          ||          ||

+---++---+  +---++---+  +---++---+  +---++---+

| Leaf 1 |  | Leaf 2 |  | Leaf 3 |  | Leaf N |

+---++---+  +---++---+  +---++---+  +---++---+

||          ||          ||          ||

[GPU/XPU]   [GPU/XPU]    [GPU/XPU]    [GPU/XPU]

Hosts w/    Hosts w/     Hosts w/     Hosts w/

RoCEv2 NIC  RoCEv2 NIC   RoCEv2 NIC   RoCEv2 NIC

The DUT boundary encompasses all leaf and spine switches and their interconnecting links.

Traffic generators or actual GPU hosts connect at the leaf layer.

3.1.2. Topology B: 3-Tier Clos (Leaf-Spine-Superspine)

For clusters exceeding thousands of accelerators, a superspine layer is added.

Each pod consists of a leaf-spine fabric, and pods are interconnected via superspine switches. This topology scales to 32,000+ accelerators at 800GbE with current-generation ASICs (e.g., current-generation deep-buffer VOQ ASICs). The DUT boundary encompasses all three tiers.

3.1.3. Topology C: Rail-Optimized

SPINE LAYER

+--------+  +--------+  +--------+  +--------+
| Spine1 |  | Spine2 |  | Spine3 |  | SpineN |
+--+--+--+  +--+--+--+  +--+--+--+  +--+--+--+

|  +--------+  +--------+  +--------+   |

|  Full Mesh Interconnect (ECMP/Spray)  |

|  +--------+  +--------+  +--------+   |

+--+--+--+  +--+--+--+  +--+--+--+  +--+--+--+
| Rail-0 |  | Rail-1 |  | Rail-2 |  | Rail-7 |   RAIL (LEAF) LAYER
| Leaf   |  | Leaf   |  | Leaf   |  | Leaf   |   one switch per NIC
+--+--+--+  +--+--+--+  +--+--+--+  +--+--+--+
  |  |        |  |        |  |        |  |
  |  |        |  |        |  |        |  |
NIC-0 NIC-0  NIC-1 NIC-1  NIC-2 NIC-2 NIC-7 NIC-7
  |  |        |  |        |  |        |  |
+--+--+--+  +--+--+--+  +--+--+--+  +--+--+--+
| Host A |  | Host B |  | Host C |  | Host D |   GPU HOSTS
| GPU[0] |  | GPU[0] |  | GPU[0] |  | GPU[0] |   (each host has
| GPU[1] |  | GPU[1] |  | GPU[1] |  | GPU[1] |    8 NICs, one
|  ...   |  |  ...   |  |  ...   |  |  ...   |    per rail)
| GPU[7] |  | GPU[7] |  | GPU[7] |  | GPU[7] |
+--------+  +--------+  +--------+  +--------+

|<-------- Rail-0 ------->|           |<- Rail-7 ->|

(all NIC-0s share Rail-0 leaf)   (all NIC-7s share Rail-7 leaf)

In rail-optimized topologies), each NIC on a multi-NIC host connects to a dedicated leaf switch ("rail"), and rails are interconnected at the spine layer. This topology co-optimizes network locality with the collective communication library (NCCL/RCCL).

The DUT boundary and rail mapping MUST be fully documented.

3.2. Device Under Test (DUT) Identification

The following parameters MUST be recorded and reported for all DUT

+--------------------+------------------------------+----------------------------+

| Parameter          | Description                  | Example                    |

+--------------------+------------------------------+----------------------------+

| Switch             | Vendor name, product family, | <Vendor> <Family> <Model>  |

| Vendor/Model       | model number                 |                            |

+--------------------+------------------------------+----------------------------+

| Switch ASIC        | Silicon vendor, ASIC family, | <Silicon Vendor>           |

|                    | revision                     | <ASIC Family> <Rev>        |

+--------------------+------------------------------+----------------------------+

| NOS Version        | Network operating system     | <NOS Name> <Version>       |

|                    | name and version             |                            |

+--------------------+------------------------------+----------------------------+

| Port Speed         | Per-port line rate           | 400GbE, 800GbE             |

+--------------------+------------------------------+----------------------------+

| Buffer             | Shared/dedicated, total      | 32MB shared +              |

| Architecture       | buffer per ASIC/port         | 16MB VOQ per port          |

+--------------------+------------------------------+----------------------------+

| Optics/Cables      | Transceiver type, cable      | OSFP 400G-DR4,             |

|                    | type and length              | DAC 3m copper              |

+--------------------+------------------------------+----------------------------+

| NIC Vendor/Model   | RDMA NIC vendor, model,      | <NIC Vendor> <Model>       |

|                    | firmware                     | <Form Factor> <Speed>      |

+--------------------+------------------------------+----------------------------+

| NIC Firmware       | NIC firmware version         | <Firmware Version>         |

+--------------------+------------------------------+----------------------------+

| Host Config        | OS, CCL lib version,         | <OS Name> <Version>,       |

|                    | driver version, BIOS         | <CCL Name> <Version>,      |

|                    | settings                     | <OFED Version>             |

+--------------------+------------------------------+----------------------------+

3.3. Traffic Generator Requirements

3.3.1. Mandatory Functional Capabilities

The traffic generator MUST be capable of emulating RoCEv2 transport including Queue Pair (QP) establishment, RDMA Write/Read operations, ECN processing, and DCQCN rate control. The generator MUST support configurable QP scaling (1 to 256 QPs per source-destination pair), programmable collective communication patterns (AllReduce, AlltoAll, AllGather with configurable message sizes), and nanosecond-precision timestamping.

3.3.2. Minimum Measurement Accuracy Requirements

Parameter                 Minimum Requirement

------------------------- -----------------------------------

Timestamp accuracy        <= 100 nanoseconds

Frame rate accuracy       +/- 0.1% of specified rate

QP scaling range          1 to 256 QPs per src-dst pair

Message size range        1 KB to 8 GB

Flow counter resolution   per-flow byte and packet counts

Loss measurement          0 ppm resolution

Burst generation          1-1000 frames at line rate

3.3.3. Acceptable Implementations

Qualifying platforms include: (a) dedicated hardware traffic generators capable of line-rate RDMA emulation with the accuracy requirements of Section 3.3.2, or (b) instrumented GPU clusters with RDMA tooling, provided the host configuration (CPU, GPU, PCIe topology, BIOS power management settings) is fully documented, as it materially affects results. When GPU clusters are used as traffic sources, any non-fabric overhead in timing measurements MUST be quantified and reported separately.

4. KPI Framework and Metrics Taxonomy

NOTE: Target values in this section are NON-NORMATIVE illustrative reference points derived from current industry practice. They do NOT constitute benchmarking acceptance criteria or performance requirements.

Per BMWG charter, defining acceptance criteria is explicitly out of scope. Implementers MAY use these values as contextual references; they MUST NOT be used as pass/fail thresholds.

This section defines a three-tier KPI hierarchy. Primary KPIs directly quantify user-facing fabric performance. Secondary KPIs provide diagnostic depth for root-cause analysis. Fabric Health Indicators track operational stability.

4.1. Primary KPIs

+--------------------+---------------+---------------------------+----------------------+

| KPI                | Unit          | Definition                | Report               |

+--------------------+---------------+---------------------------+----------------------+

| Job Completion     | seconds       | Wall-clock time for       | Minimize; report     |

| Time (JCT)         |               | benchmark iteration       | as absolute and      |

|                    |               | including compute +       | as JCT Ratio vs.     |

|                    |               | communication phases      | Roofline             |

+--------------------+---------------+---------------------------+----------------------+

| JCT Ratio          | dimensionless | Measured JCT / Roofline   | <= 1.05 (better than |

|                    |               | JCT; fabric overhead      | <= 1.15  OK)         |

|                    |               | factor                    |                      |

+--------------------+---------------+---------------------------+----------------------+

| Bus Bandwidth      | Gbps per      | Effective per-accelerator | >= 90% of NIC        |

| (BusBW)            | accelerator   | throughput during         | line rate for        |

|                    |               | collective operation      | intra-pod traffic    |

+--------------------+---------------+---------------------------+----------------------+

| Aggregate          | Tbps          | Total fabric goodput      | >= 95% of            |

| Throughput         |               | during collective         | bisection BW         |

|                    |               | communication phase       |                      |

+--------------------+---------------+---------------------------+----------------------+

| Packet Drop Rate   | ppm (parts    | Frames lost end-to-end    | 0 ppm (lossless)     |

|                    | per million)  | not retransmitted         | report any non       |                      |                    |               |			     | Value                |

+--------------------+---------------+---------------------------+----------------------+

| Tail Latency       | microseconds  | 99th / 99.9th percentile  | Report absolute;     |

| (P99 / P99.9)      |               | one-way fabric latency    | minimize             |

+--------------------+---------------+---------------------------+----------------------+

4.2. Secondary KPIs

+---------------------------+-----------------+------------------------------------------+
| KPI                       | Unit            | Definition                              |
+---------------------------+-----------------+------------------------------------------+
| ECN Marking Ratio         | %               | Percentage of packets marked CE          |
|                           |                 | over measurement interval                |
+---------------------------+-----------------+------------------------------------------+
| PFC Pause Count           | events/sec      | Rate of PFC PAUSE frames generated       |
|                           |                 | per priority per port                    |
+---------------------------+-----------------+------------------------------------------+
| PFC Pause Duration        | microseconds    | Cumulative time a port spends in         |
|                           |                 | PFC-paused state per interval            |
+---------------------------+-----------------+------------------------------------------+
| RDMA Retransmission Rate  | retx/sec        | NIC-level retransmissions due to         |
|                           |                 | timeouts or NAKs                         |
+---------------------------+-----------------+------------------------------------------+
| ECMP Imbalance (MMR)      | dimensionless   | Max-Mean Ratio of flow counts            |
|                           |                 | across parallel uplinks                  |
+---------------------------+-----------------+------------------------------------------+
| Jain Fairness Index (JFI) | 0.0 - 1.0       | Fairness of traffic distribution         |
|                           |                 | across fabric links; 1.0 = perfect       |
+---------------------------+-----------------+------------------------------------------+
| Queue Depth (P95/Max)     | bytes or cells  | 95th percentile and maximum egress       |
|                           |                 | queue occupancy per port                 |
+---------------------------+-----------------+------------------------------------------+
| Congestion Control        | microseconds    | Time from congestion onset to            
| Convergence               |                 | DCQCN rate stabilization                 |
+---------------------------+-----------------+------------------------------------------+
| Out-of-Order Packet Rate  | pkt/sec         | Packets delivered out of sequence;       |
|                           |                 | relevant for packet spray configurations |
+---------------------------+-----------------+------------------------------------------+
| CTS/ACK Delay             | microseconds    | Delay for control messages               |
|                           |                 | (Clear-to-Send, ACKs) in                 |
|                           |                 | receiver-driven schemes                  |
+---------------------------+-----------------+------------------------------------------+

4.3. Fabric Health Indicators

+---------------------------+--------------+------------------------------------------+

| Indicator                 | Unit         | Definition                               |

+---------------------------+--------------+------------------------------------------+

| Switch CPU Utilization    | %            | Average and peak CPU usage on DUT        |

|                           |              | control plane during test                |

+---------------------------+--------------+------------------------------------------+

| Switch Memory             | %            | Average and peak memory usage,           |

| Utilization               |              | including FIB/MAC table occupancy        |

+---------------------------+--------------+------------------------------------------+

| FIB/Route Convergence     | ms           | Time to converge routing after           |

| Time                      |              | topology change                          |

+---------------------------+--------------+------------------------------------------+

| Link Flap Count           | events       | Spurious link state changes during       |

|                           |              | test period                              |

+---------------------------+--------------+------------------------------------------+

| CRC/FCS Error Rate        | errors/sec   | Physical layer errors indicating         |

|                           |              | cable or optics issues                   |

+---------------------------+--------------+------------------------------------------+

| Power Consumption         | Watts        | Per-switch and per-port power draw       |

|                           |              | under test load                          |

+---------------------------+--------------+------------------------------------------+

5. Test Category 1: RDMA Transport Benchmarks

These tests establish baseline fabric performance for RDMA traffic independent of collective communication patterns. They extend RFC 2544 and RFC 8239 methodology for RoCEv2 semantics.

5.1. Baseline Throughput

Objective: Determine the maximum sustainable RDMA Write throughput through the DUT fabric at each tested message size, analogous to RFC 2544 throughput testing but using RoCEv2 transport.

Procedure: Configure N host pairs, each establishing Q Queue Pairs per pair. Initiate RDMA Write operations with the specified message size and measure aggregate goodput. The test MUST be run for at least 60 seconds at each rate. A binary search (per RFC 2544 Section 26.1) SHOULD be used to find the maximum rate with zero packet loss. The throughput MUST be measured for the following message sizes: 64B, 256B, 1KB, 4KB, 64KB, 256KB, 1MB, 4MB. The test MUST be repeated for QP counts of 1, 4, 16, and 32 per source-destination pair. Test with both unidirectional and bidirectional traffic.

Reporting: Report aggregate throughput (Tbps), per-port utilization (%), and throughput efficiency (measured/theoretical). Present results as a table indexed by message size and QP count, and as a graph with message size on the X-axis and throughput on the Y-axis.

5.2. Latency Characterization

Objective: Determine the one-way and round-trip RDMA latency distribution through the DUT fabric at the throughput rate determined in Section 5.1.

Procedure: At the determined throughput rate for each message size, inject tagged frames at 60 seconds into a 120-second stream (per RFC 2544 Section 26.2). Record timestamps at nanosecond precision. The test MUST report minimum, mean, P50, P95, P99, P99.9, and maximum latency values. Repeat at least 20 times and report the average of recorded values. Conduct tests under both zero-load (single QP, single flow) and loaded (full fabric utilization) conditions to characterize the latency increase under contention.

Reporting: Tabulate latency statistics for each message size. Additionally, provide a latency histogram and a CDF plot. Report the latency increase factor (loaded/unloaded) as a key diagnostic metric.

5.3. Back-to-Back Burst Absorption

Objective: Characterize the DUT fabric's ability to absorb back-to-back RDMA bursts without loss, extending RFC 9004 methodology for RoCEv2.

Procedure: Transmit bursts of RDMA Write frames at line rate with minimum inter-frame gap. Increase burst length until the first frame loss is detected. The back-to-back value is the maximum burst length (in bytes and frames) handled without loss. Test at incast ratios of 2:1, 4:1, 8:1, 16:1, and 32:1. Repeat at least 50 times per burst length and report the average.

This test directly measures the effective buffer depth available for absorbing AI training microbursts.

Reporting: Report burst absorption capacity in frames and bytes for each message size and incast ratio. Plot burst capacity vs. incast ratio to characterize the buffer utilization efficiency.

6. Test Category 1A: UEC Transport Protocol Benchmarks

The Ultra Ethernet Consortium (UEC) Specification 1.0 defines the Ultra

Ethernet Transport (UET) protocol, a connectionless RDMA transport designed to replace RoCEv2 for AI/HPC workloads. UET introduces four transport services (ROD, RUD, RUDI, UUD), native per-packet multipath spraying without reorder buffer requirements (via RUD), sender-plus-receiver congestion control, and optional link-layer enhancements (LLR, Packet Trimming, CBFC). These tests benchmark UET-specific behaviors and provide direct comparative measurements against RoCEv2 on identical topologies. All UET tests use the libfabric API over UET and run on UEC 1.0-compliant NICs.

The UEC compliance profile (AI Base, AI Full, or HPC) used during testing MUST be documented.

6.1. UET Throughput by Transport Service

Objective: Determine the maximum sustainable throughput achievable under each UET transport service (ROD, RUD, RUDI, UUD) and compare to RoCEv2 RC/UC modes on the same DUT fabric.

Procedure: Configure N host pairs using UEC 1.0-compliant NICs. For each transport service (ROD, RUD, RUDI, UUD), establish Packet Delivery Contexts (PDCs) and initiate bulk data transfers using the libfabric fi_write (RDMA Write equivalent) verb. Measure aggregate goodput at each message size: 64B, 256B, 1KB, 4KB, 64KB, 256KB, 1MB, 4MB. Apply the binary search procedure from RFC 2544 Section 26.1 to find the maximum zero-loss throughput. The test MUST be run for at least 60 seconds per trial. PDC counts per source-destination pair MUST be varied: 1, 4, 16, 32 (analogous to QP count variation in Section 5.1). A parallel RoCEv2 test series using the same host pairs, message sizes, and QP counts MUST be executed for direct comparison. Both unidirectional and bidirectional configurations MUST be tested.

Reporting: Present a comparison table indexed by (transport_service, message_size, PDC_count) with columns for UET throughput (Gbps), RoCEv2 throughput (Gbps), and delta (%). Plot throughput vs. message size for each transport service overlaid with RoCEv2 RC baseline. Report

throughput efficiency = measured / theoretical_line_rate.

+---------------------+---------+---------+---------+---------+-----------+-----------+

| Metric              | ROD     | RUD     | RUDI    | UUD     | RoCEv2 RC | RoCEv2 UC |

+---------------------+---------+---------+---------+---------+-----------+-----------+

| Throughput @ 1MB    | (meas)  | (meas)  | (meas)  | (meas)  | (meas)    | (meas)    |

| (Gbps)              |         |         |         |         |           |           |

+---------------------+---------+---------+---------+---------+-----------+-----------+

| Throughput @ 4MB    | (meas)  | (meas)  | (meas)  | (meas)  | (meas)    | (meas)    |

| (Gbps)              |         |         |         |         |           |           |

+---------------------+---------+---------+---------+---------+-----------+-----------+

| Efficiency          | (meas)  | (meas)  | (meas)  | (meas)  | (meas)    | (meas)    |

| (% line rate)       |         |         |         |         |           |           |

+---------------------+---------+---------+---------+---------+-----------+-----------+

| PDC/QP Setup        | (meas)  | (meas)  | (meas)  | (meas)  | (meas)    | (meas)    |

| Time (us)           |         |         |         |         |           |           |

+---------------------+---------+---------+---------+---------+-----------+-----------+

| Max Sustained       | (meas)  | (meas)  | (meas)  | (meas)  | (meas)    | (meas)    |

| PDC/QP Count        |         |         |         |         |           |           |

+---------------------+---------+---------+---------+---------+-----------+-----------+

6.2. UET Latency Characterization

Objective: Measure the one-way and round-trip latency distribution for UET transport services and quantify any latency differential versus RoCEv2, with particular attention to connectionless PDC establishment overhead.

Procedure: At the determined throughput rate from Section 6.1 for each message size, inject tagged frames at 60 seconds into a 120-second stream per RFC 2544 Section 26.2. Record timestamps at nanosecond precision.

Report minimum, mean, P50, P95, P99, P99.9, and maximum latency values. Repeat at least 20 times and average. Measure latency for: (a) steady-state PDC transfers (PDC already established), (b) first-packet latency (PDC establishment + first data packet, measuring UET's "data before handshake" optimization), (c) zero-load baseline (single PDC, single host pair). Test under ROD (ordered) and RUD (unordered) services separately to isolate reordering-related latency components.

Reporting: Tabulate latency statistics for each (transport_service, message_size, load_condition) tuple. Plot latency CDF for UET ROD, UET RUD, and RoCEv2 RC side-by-side. Report the first-packet latency advantage of UET's connectionless model over RoCEv2 QP establishment (which requires a 3-way handshake).

6.3. Packet Spray Efficacy Under UET RUD

Objective: Quantify the load balancing improvement achieved by UET's native per-packet spray with RUD (Reliable Unordered Delivery), which eliminates the receiver reorder buffer requirement that constrains packet spray effectiveness under RoCEv2.

Procedure: Configure the DUT fabric with per-packet load balancing (spray). Generate bulk transfer traffic from N source hosts to N destination hosts using: (a) UET RUD with per-packet spray (NIC provides per-packet entropy values), (b) UET ROD with per-packet spray (NIC or switch provides reordering), (c) RoCEv2 RC with per-packet spray (switch fabric provides reorder buffering), (d) RoCEv2 RC with standard ECMP.

For each configuration, measure: Max-Mean Ratio (MMR) across all fabric uplinks, Jain Fairness Index (JFI), out-of-order packet delivery rate at the destination NIC, RDMA/UET retransmission rate, and effective goodput after retransmissions. Vary the number of ECMP paths (4, 8, 16, 32) and host pair counts (8, 16, 32, 64). Measure the per-packet entropy distribution provided by the UEC NIC's entropy value field.

Reporting: Present a comparison table for each (spray_mode, path_count, host_pairs) combination. Plot JFI and MMR for UET RUD vs. RoCEv2 RC spray. The key result is the out-of-order rate: UET RUD SHOULD achieve zero host-visible reordering despite per-packet spray because the transport layer natively tolerates unordered delivery. Report the entropy value distribution to verify uniform ECMP path utilization.

+----------------------+--------+--------+--------+--------+------------------+

| Load Balancing       | MMR    | JFI    | OOO    | Retx   | Effective        |

| Config               |        |        | Rate   | Rate   | Goodput (%)      |

+----------------------+--------+--------+--------+--------+------------------+

| UET RUD +            | (meas) | (meas) | (meas) | (meas) | (meas)           |

| Packet Spray         |        |        |        |        |                  |

+----------------------+--------+--------+--------+--------+------------------+

| UET ROD +            | (meas) | (meas) | (meas) | (meas) | (meas)           |

| Packet Spray         |        |        |        |        |                  |

+----------------------+--------+--------+--------+--------+------------------+

| RoCEv2 RC +          | (meas) | (meas) | (meas) | (meas) | (meas)           |

| Packet Spray         |        |        |        |        |                  |

+----------------------+--------+--------+--------+--------+------------------+

| RoCEv2 RC +          | (meas) | (meas) | (meas) | (meas) | (meas)           |

| ECMP (baseline)      |        |        |        |        |                  |

+----------------------+--------+--------+--------+--------+------------------+

| UET RUD +            | (meas) | (meas) | (meas) | (meas) | (meas)           |

| DLB/Flowlet          |        |        |        |        |                  |

+----------------------+--------+--------+--------+--------+------------------+

6.4. UET Congestion Control Benchmarks

Objective: Evaluate UET's dual-sided (sender + receiver) congestion control mechanism under N:1 incast conditions and compare convergence behavior to RoCEv2 DCQCN.

Procedure: UET specifies a coordinated congestion control architecture with both sender-based rate control (driven by ECN feedback) and receiver-based credit allocation (which proactively throttles senders during incast). This test measures: (a) Incast throughput: generate N:1 incast (N = {2, 4, 8, 16, 32, 64}) using UET RUD and measure aggregate throughput at the receiver vs. the same topology with RoCEv2 DCQCN. (b)

Convergence time: at time T0, double the number of active senders from M to 2M through a shared bottleneck and measure the time until all flows achieve rates within 10% of fair share. Record peak queue depth and any PFC/CBFC events generated. (c) PFC avoidance: UET is designed to operate in a lossy Ethernet environment without depending on PFC. Measure the packet drops rate and retransmission overhead under identical incast conditions with PFC disabled on the DUT. Compare with RoCEv2, which requires PFC for lossless operation. (d) Receiver credit utilization: measure the receiver credit allocation efficiency under incast (ratio of utilized credits to allocated credits).

Reporting: Tabulate incast throughput, convergence time, peak queue depth, PFC event count, and packet drop rate for UET vs. DCQCN at each incast ratio. Plot per-flow rate vs. time during convergence for both protocols. The critical differentiator is the PFC-free operation test report whether UET achieves zero application-visible loss without PFC and the retransmission overhead cost.

6.5. Link Layer Enhancement Benchmarks

Objective: Measure the performance impact of UEC optional link-layer enhancement Link Layer Retry (LLR), Packet Trimming (PT), and Credit-Based Flow Control (CBFC).

Procedure: These tests require both the DUT switch and the NICs to support the respective optional features. For each enhancement, conduct baseline (disabled) and enhanced (enabled) runs:(a) LLR Retry Latency: inject controlled bit errors on a fabric link using a test attenuator or error injector. Measure the LLR retry success rate, LLR retry latency (expected sub-microsecond per hop), and compare to the transport-layer retransmission latency incurred without LLR (expected ~10-100us RTT). Run with background traffic at 80%-line rate. (b) Packet Trimming Effectiveness: configure a bottleneck link at 2:1 oversubscription. With PT enabled, congested switches transmit packet headers only (trimmed packets) instead of dropping. Measure: (i) the time from congestion onset to first retransmission request (PT should accelerate loss detection by delivering the header/metadata to the receiver even under congestion), (ii) the bandwidth saved by transmitting trimmed headers vs. full packets during congestion, (iii) the end-to-end retransmission latency with and without PT. (c) CBFC vs. PFC: configure identical incast scenarios (N:1, N=32) with CBFC enabled (UET) vs. PFC enabled (RoCEv2). Measure: (i) head-of-line blocking duration (CBFC is per-destination, PFC is

per-priority), (ii) pause propagation hops (CBFC should not propagate beyond one hop), (iii) throughput of non-congested flows sharing the same port (CBFC should preserve their bandwidth).

Reporting: For each enhancement, present a before/after comparison table. Report LLR retry latency distribution (min, P50, P99, max). Plot trimming bandwidth savings vs. congestion severity. Tabulate CBFC vs. PFC metrics. Note which features are hardware-supported vs. software-emulated on the tested NIC/switch.

6.6. UET Collective Communication Performance

Objective: Measure collective communication (AllReduce, AlltoAll, AllGather) performance over UET and compare to RoCEv2, isolating the transport protocol's contribution to collective efficiency.

Procedure: Using a UEC-compliant collective library (e.g., RCCL with UET backend, or libfabric-based collective), execute the identical collective benchmark suite from Section 9 (AllReduce, AlltoAll, AllGather) over UET RUD transport. The same accelerator count (N), message sizes (S), and fabric topology MUST be used. Additionally, for AllReduce specifically, leverage UET's group keying feature (transport-layer multicast/reduction support) if available on the DUT NIC.

Measure BusBW, JCT per iteration, ECN marking ratio, and PFC/CBFC event count. Run under UET RUD + packet spray as the primary configuration, and UET ROD + ECMP as a secondary baseline.

Reporting: Present a direct comparison table: UET RUD BusBW vs. RoCEv2 RC BusBW for each (collective, message_size, N) combination. Plot BusBW vs. N for AllReduce over UET RUD, UET ROD, and RoCEv2 RC. Report the percentage improvement in BusBW and JCT attributable to UET's native spray tolerance and congestion control.

---------------------------------------------------------------------------------------

**Collective**   **Msg    **N        **UET RUD   **UET ROD   **RoCEv2 RC  **Delta

Size**   Accels**   BusBW**     BusBW**     BusBW**      UET/RoCEv2**

---------------- -------- ---------- ----------- ----------- ------------ -------------

AllReduce        1GB      128        (meas)      (meas)      (meas)       (meas)

AllReduce        1GB      512        (meas)      (meas)      (meas)       (meas)

AlltoAll         1GB      128        (meas)      (meas)      (meas)       (meas)

AllGather        1GB      128        (meas)      (meas)      (meas)       (meas)

---------------------------------------------------------------------------------------

6.7. UET PDC Scalability and Connection Setup Rate

Objective: Measure the rate at which UET Packet Delivery Contexts (PDCs) can be established and the maximum concurrent PDC count, benchmarking the connectionless model's scalability advantage over RoCEv2 QP-based connections.

Procedure: (a) PDC establishment rate: initiate PDC creation in rapid succession from a single NIC to M remote endpoints (M = {100, 1000, 10000, 100000}) and measure the sustained PDC creation rate (PDCs/second). For comparison, measure QP creation rate on the same NIC under RoCEv2. (b) Data-before-handshake: UET allows the sender to transmit data in the first packet before the PDC handshake completes.

Measure the first-byte latency (time from fi_write call to first byte on the wire) for UET vs. RoCEv2 RDMA Write (which requires QP establishment first). (c) Maximum concurrent PDC count: progressively increase the number of active PDCs on a single NIC until performance degrades (throughput per PDC drops below 90% of the single-PDC rate). The UEC specification targets up to 1 million endpoints; verify the practical limit on the DUT NIC.

Reporting: Report PDC establishment rate vs. QP establishment rate. Plot first-byte latency comparison. Report the maximum concurrent PDC count with its associated per-PDC throughput.

7. Test Category 2: Congestion Management Benchmarks

AI training workloads generate repetitive micro-congestion during the back-propagation gradient synchronization phase. These tests evaluate how effectively the DUT fabric manages this congestion using ECN, PFC, and DCQCN mechanisms.

7.1. ECN Marking Accuracy and Threshold

Objective: Verify that the DUT marks packets with ECN Congestion Experienced (CE) at the configured threshold and with the correct granularity.

Procedure: Configure an ECN marking threshold T (in bytes or cells) on the DUT egress queue. Generate traffic to create a controlled queue buildup at an oversubscription ratio of 1.1:1. Capture all packets at the receiver and compute the ECN marking ratio as a function of measured queue depth. The test MUST verify that (a) no packets are ECN-marked below threshold T, (b) 100% of packets are ECN-marked above the maximum threshold, and (c) the marking probability ramps appropriately between thresholds (per WRED/RED configuration). Repeat for multiple ECN threshold configurations: low (e.g., 100KB), medium (e.g., 1MB), high (e.g., 5MB).

Reporting: Plot ECN marking probability vs. instantaneous queue depth.

Report the measured threshold accuracy (deviation from configured

threshold).

7.2. PFC Behavior Under Incast

Objective: Characterize the DUT's PFC generation behavior under N:1 incast conditions representative of AI training AllReduce operations.

Procedure: Configure PFC on the DUT with headroom and threshold values appropriate for the link speed and cable length. Generate N:1 incast traffic at 100%-line rate from N sources to 1 destination, where N = {2, 4, 8, 16, 32, 64}. Measure: (a) PFC PAUSE frame count per second at each hop, (b) PFC PAUSE duration per port, (c) whether PFC propagates upstream ("PFC storm" onset), (d) end-to-end throughput achieved. The test SHOULD verify that PFC headroom is correctly sized (no drops despite PFC activation) and that PFC watchdog mechanisms prevent persistent pauses.

Reporting: Tabulate PFC metrics vs. incast ratio. Plot PFC pause count and throughput vs. incast ratio. Report any packet loss observed during PFC operation.

7.3. DCQCN Convergence Time

Objective: Measure the time for the DCQCN congestion control loop to converge to a fair-share rate after congestion onset.

Procedure: Establish M flows through a common bottleneck link. At time T0, inject an additional M flow to create 2:1 oversubscription. Measure the time from T0 until all 2M flows achieve rates within 10% of fair share. Record the peak queue depth during convergence and any PFC PAUSE events generated. Repeat for M = {4, 16, 64, 256}. Vary DCQCN parameters (alpha, beta, rate reduction factor, rate increase timer) to characterize sensitivity.

Reporting: Report convergence time for each M value. Plot per-flow rate vs. time to visualize the convergence dynamics.

7.4. PFC Storm and Deadlock Resilience

Objective: Verify that the DUT does not enter a PFC deadlock or sustained PFC storm under adversarial traffic patterns.

Procedure: Generate cyclic traffic patterns known to cause PFC deadlocks in vulnerable implementations (e.g., circular dependencies across fabric tiers). Run for 300 seconds. Monitor for: (a) PFC PAUSE frames exceeding the configured watchdog threshold, (b) complete cessation of traffic on any port/priority, (c) PFC watchdog trigger events. The DUT MUST demonstrate resilience: either preventing the deadlock via PFC watchdog (dropping offending traffic) or via architectural immunity (e.g., VOQ-based scheduling).

Reporting: Report whether any PFC storm or deadlock was observed. Report PFC watchdog activation events and their impact on traffic.

8. Test Category 3: Load Balancing Efficacy

Load balancing across parallel fabric paths is critical for AI training fabrics because the traffic consists of a small number of high-bandwidth, long-lived elephant flows. Traditional 5-tuple ECMP performs poorly in this regime due to low flow entropy. These tests quantify load balancing effectiveness.

8.1. ECMP Entropy and Polarization

Objective: Quantify the degree of traffic polarization (uneven distribution) under standard ECMP hashing for AI training flow patterns.

Procedure: Configure the DUT with standard 5-tuple ECMP. Generate traffic with realistic AI training flow characteristics: F flows from each of N leaf switches to each of N leaf switches, using Q QPs per source-destination pair (Q = {1, 4, 8, 16, 32}). Measure the per-link utilization on all uplinks between leaf and spine tiers. Compute the Max-Mean Ratio (MMR) and Jain Fairness Index (JFI). Test with and without BTH-aware hashing (parsing RoCEv2 Base Transport Header fields for additional entropy). Repeat for fabric sizes of 8, 16, 32, and 64 leaf switches.

Reporting: Report MMR and JFI for each QP count and fabric size. Tabulate per-link utilization distribution (min, P10, P50, P90, max).

8.2. Dynamic Load Balancing (Flowlet)

Objective: Evaluate the DUT's flowlet-based DLB performance for AI training traffic and compare to baseline ECMP.

Procedure: Configure the DUT with vendor-specific DLB (document DLB algorithm type in DUT configuration). Generate the same traffic pattern as Section 8.1 with Q=4 QPs. Measure MMR, JFI, per-link utilization, and out-of-order packet rate. Vary the flowlet gap timer and report sensitivity. Compare throughput and JCT (if collective-level testing is used) to baseline ECMP.

Reporting: Report MMR, JFI, throughput improvement over ECMP (%), out-of-order rate, and flowlet gap sensitivity analysis.

8.3. Packet Spraying

Objective: Evaluate the DUT's per-packet spraying performance and quantify the tradeoff between utilization and packet reordering.

Procedure: Configure the DUT for per-packet load balancing (packet spray). Generate the same traffic pattern as Section 8.1. Measure MMR (expected near 1.0), JFI (expected near 1.0), out-of-order packet rate, and the impact on RDMA retransmissions (since RoCEv2 RC mode is sensitive to reordering). If the DUT provides reorder buffer functionality (if the DUT provides an in-fabric reorder buffer, document this capability per Appendix B), verify that out-of-order delivery to the host is zero.

Reporting: Report all metrics from Section 8.1 plus out-of-order rate, retransmission rate, and effective throughput after retransmissions.

8.4. Jain Fairness Index Measurement

Objective: Provide a single-number summary of load balancing quality comparable across all strategies.

Procedure: For each load balancing strategy tested in Sections 7.1-7.3, compute the Jain Fairness Index (JFI) as: JFI = (sum(LinkTx_i))^2 / (N * sum(LinkTx_i^2)), where LinkTx_i is the transmitted traffic on fabric link i over the measurement interval, and N is the total number of parallel links. JFI ranges from 1/N (worst case: all traffic on one link) to 1.0 (perfect distribution).

Reporting: Report JFI for each load balancing strategy. Provide a bar chart comparing JFI across ECMP, DLB, and packet spray.

9. Test Category 4: Collective Communication Benchmarks

These tests evaluate the fabric's performance under realistic collective communication patterns generated by AI training frameworks.

Unlike the synthetic RDMA tests in Section 5 and Section 6, these tests exercise the full communication stack including the collective library (NCCL, RCCL, or equivalent) and its interaction with the network.

9.1. AllReduce Benchmark

Objective: Measure fabric performance during AllReduce operations, the dominant collective for gradient synchronization in data-parallel training.

Procedure: Using N accelerators connected through the DUT fabric, execute AllReduce (sum) operations using the collective communication library benchmark suite (e.g., nccl-tests or equivalent). Test with message sizes of 1MB, 8MB, 64MB, 256MB, 1GB, and 4GB. Vary the number of participating accelerators: N = {8, 16, 32, 64, 128, 256, 512, 1024}. For each (message_size, N) pair, execute at least 100 iterations and report the average, P50, P95, P99 BusBW. Record ECN marking ratio, PFC pause count, and per-link utilization during the test. Run under each load balancing strategy (ECMP, DLB, spray).

Reporting: Tabulate BusBW for each (message_size, N, LB_strategy) combination. Plot BusBW vs. N for each message size. Report BusBW efficiency = BusBW / NIC_line_rate.

9.2. AlltoAll Benchmark

Objective: Measure fabric performance during AlltoAll operations, used in Mixture-of-Experts (MoE) models and expert parallelism.

Procedure: Execute AlltoAll operations via the collective communication library with the same message size and N permutations as Section 9.1. AlltoAll creates the worst-case congestion scenario because every accelerator simultaneously sends data to every other accelerator, generating maximum entropy and maximum incast. Measure BusBW, JCT per iteration, ECN ratio, PFC count, and fabric link utilization.

Reporting: Same format as Section 9.1. Additionally, report the JCT for AlltoAll iterations, as AlltoAll JCT is the most sensitive indicator of fabric congestion management quality.

9.3. AllGather Benchmark

Objective: Measure fabric performance during AllGather operations, used for parameter distribution in tensor-parallel and pipeline-parallel training.

Procedure: Execute AllGather operations via the collective communication library with the same parameter sweeps as Section 9.1. Measure BusBW, JCT, and fabric health indicators.

Reporting: Same format as Section 9.1.

9.4. Collective Communication Library Bus Bandwidth Summary

Objective: Provide a consolidated view of fabric-level collective performance across all operations.

Reporting: Present a summary table with the following structure:

+----------------------+--------+--------+--------+--------+------------------+

| Load Balancing       | MMR    | JFI    | OOO    | Retx   | Effective        |

| Config               |        |        | Rate   | Rate   | Goodput (%)      |

+----------------------+--------+--------+--------+--------+------------------+

| UET RUD +            | (meas) | (meas) | (meas) | (meas) | (meas)           |

| Packet Spray         |        |        |        |        |                  |

+----------------------+--------+--------+--------+--------+------------------+

| UET ROD +            | (meas) | (meas) | (meas) | (meas) | (meas)           |

| Packet Spray         |        |        |        |        |                  |

+----------------------+--------+--------+--------+--------+------------------+

| RoCEv2 RC +          | (meas) | (meas) | (meas) | (meas) | (meas)           |

| Packet Spray         |        |        |        |        |                  |

+----------------------+--------+--------+--------+--------+------------------+

| RoCEv2 RC +          | (meas) | (meas) | (meas) | (meas) | (meas)           |

| ECMP (baseline)      |        |        |        |        |                  |

+----------------------+--------+--------+--------+--------+------------------+

| UET RUD +            | (meas) | (meas) | (meas) | (meas) | (meas)           |

| DLB/Flowlet          |        |        |        |        |                  |

+----------------------+--------+--------+--------+--------+------------------+

10. Test Category 5: Job Completion Time (JCT) Benchmarks

JCT is the single most important user-facing KPI for AI training fabrics. It directly determines accelerator utilization and training cost. These tests measure JCT under controlled conditions to enable fabric-to-fabric comparison.

10.1. Synthetic JCT Under Controlled Conditions

Objective: Measure the JCT for a defined synthetic workload that exercises the fabric with a known computation-to-communication ratio, enabling isolation of fabric-induced overhead.

Procedure: Define a synthetic training iteration as: (1) a computation phase of C milliseconds (simulated by a sleep or GPU compute kernel), followed by (2) a communication phase consisting of an AllReduce of S bytes across N accelerators. Vary C in {10ms, 50ms, 100ms, 500ms}, S in {256MB, 1GB, 4GB}, and N in {64, 128, 256, 512, 1024}. Execute 1000 iterations and measure the total wall-clock JCT. Compute the Roofline

JCT as: Roofline = Iterations * (C + S * algo_factor / NIC_line_rate).

Report the JCT Ratio = Measured_JCT / Roofline_JCT.

Reporting: Tabulate JCT Ratio for each (C, S, N, LB_strategy) combination. Plot JCT Ratio vs. N to characterize fabric scalability. A JCT Ratio below 1.05 represents excellent fabric performance in current deployments; above 1.15 represents significant fabric-induced overhead. NOTE: These are non-normative illustrative reference values only.

10.2. MLPerf-Aligned JCT

Objective: Measure JCT using MLPerf Training benchmark workloads to enable comparison with published industry results.

Procedure: Execute MLPerf Training benchmark closed-division workloads (e.g., BERT, ResNet, GPT-3 175B) on the DUT fabric according to MLPerf submission rules. Measure time-to-train (the MLPerf primary metric) and simultaneously capture all fabric KPIs from Section 4. If full MLPerf submission is not feasible, use MLPerf-equivalent microbenchmarks (e.g., the collective library benchmark suite with message sizes and collective patterns matching the target model).

Reporting: Report time-to-train and/or tokens-per-second as primary results. Report all fabric KPIs as secondary diagnostics. Compare across load balancing strategies.

10.3. Multi-Tenant JCT Interference

Objective: Quantify the impact on JCT when multiple training jobs share the same fabric (multi-tenancy).

Procedure: Configure two or more independent training jobs on the same DUT fabric, with each job assigned a subset of accelerators. Jobs SHOULD overlap in their use of spine-layer links. Measure JCT for each job independently (baseline), then simultaneously (contention). The JCT interference factor = Contention_JCT / Baseline_JCT. Test with varying degrees of overlap (0%, 25%, 50%, 75% of spine links shared).

Reporting: Report the JCT interference factor for each job and each overlap level. This test is critical for cloud providers and shared-infrastructure operators.

11. Test Category 6: Scale and Convergence

11.1. Fabric Scale Limits

Objective: Determine the maximum fabric scale at which the DUT maintains acceptable KPI performance.

Procedure: Progressively increase the number of active accelerator endpoints from N=64 to the maximum supported by the topology while running the AllReduce benchmark from Section 9.1 with S=1GB. At each scale point, record JCT Ratio, BusBW, ECN ratio, PFC count, CPU utilization, and memory utilization. The scale limit is the largest N at which the JCT Ratio and packet drop rate are fully characterized and reported. Additionally, measure BGP/routing convergence time after clearing all adjacencies, analogous to the EVPN scale test methodology defined in [EVPN-BENCH] Section 6.

Reporting: Plot all KPIs vs. N. Report the determined scale limit.

11.2. Link Failure Convergence

Objective: Measure the traffic disruption and JCT impact when a fabric link fails during active training.

Procedure: With the fabric fully loaded (AllReduce, N=128, S=1GB), administratively fail a spine uplink. Measure: (a) the duration of packet loss (if any), (b) the number of packets lost, (c) the JCT overhead for the iteration in which the failure occurs compared to steady state, (d) the time for the load balancing mechanism to redistribute flows. Repeat for leaf uplink failure, spine switch failure, and superspine link failure (if applicable). Test under each load balancing strategy.

Reporting: Report failover time (microseconds), packets lost, and JCT overhead (%) for each failure scenario and LB strategy.

11.3. Zero-Impact Failover Measurement

Objective: Verify vendor claims of zero-impact or sub-microsecond failover for AI training traffic.

Procedure: Execute the test from Section 11.2 with nanosecond-precision measurement. A failure is considered "zero-impact" if the measured JCT for the failure iteration is within the P99 JCT of steady-state iterations (i.e., the failure is not statistically distinguishable from normal variation).

Report the actual failover time and whether the zero-impact criterion is met.

12. Test Category 7: Soak and Stability

12.1. 24-Hour Sustained Load

Objective: Verify DUT fabric stability under sustained AI training load over an extended period, following the methodology pattern from [EVPN-BENCH] Section 7.

Procedure: Configure the DUT at maximum validated scale from Section 11.1. Generate bidirectional collective communication traffic (alternating AllReduce and AlltoAll) with F frames per second in each direction, at N EVIs/VRFs as appropriate. Run continuously for 24 hours.

Sample all KPIs from Section 4 every 60 seconds. Record: (a) any JCT Ratio excursion above 1.15, (b) any packet drops, (c) any PFC storm events, (d) any route withdrawals or BGP flaps, (e) CPU and memory utilization trends.

Reporting: Provide time-series plots of JCT Ratio, BusBW, ECN ratio, PFCcount, CPU, and memory over the 24-hour period. Report the standard deviation of JCT Ratio (stability metric). There SHOULD NOT be any memory leaks, crashes, or CPU spikes. Any anomaly MUST be reported with timestamp and duration.

12.2. Resource Leak Detection

Objective: Detect memory leaks, handle exhaustion, or gradual performance degradation in DUT software.

Procedure: Record per-process memory usage at T=0, T=1h, T=6h, T=12h, and T=24h. Compute the linear regression slope of memory usage over time for each monitored process. A slope exceeding 1MB/hour for any process indicates a potential memory leak and MUST be reported. Additionally, monitor forwarding-plane counter wraparounds and hardware table (FIB, MAC, ARP/ND) occupancy trends.

13. Reporting Format

Test reports MUST include the following sections:

- DUT Identification: Complete parameters from Section 3.2 for allfabric component.

- Test Topology: Diagram and description per Section 3.1, including physical cabling.

3. Test Configuration: All DUT configuration parameters including QoS policies (ECN thresholds, PFC headroom, DCQCN parameters), load balancing mode, buffer allocation, and any vendor-specific tuning

applied.

4. Host Configuration: Complete host stack description per Section 3.2 including NIC firmware, driver, collective library version, and any tuning applied (e.g., collective algorithm selection, protocol variant, GPU Direct settings). For UET tests, additionally

report: UEC compliance profile (AI Base, AI Full, or HPC), libfabric provider version, NIC UEC firmware version, and which optional link-layer features are enabled (LLR, Packet Trimming, PRI, CBFC).

5. Test Results: For each test from Sections 5-12, provide the specified reporting format including tables, graphs, and statistical summaries. For Section 6 (UEC) tests, results MUST include side-by-side UET vs. RoCEv2 comparison data on the identical DUT fabric.

6. Anomalies: Any deviations from the specified procedures, test failures, or unexpected behaviors MUST be documented.

7. Repeatability Statement: For each test, report the number of iterations performed and the coefficient of variation (standard deviation / mean) for the primary metric. A CV below 5% is RECOMMENDED for test validity.

14. Security Considerations

This document defines benchmarking methodologies for controlled laboratory environments and does not introduce new security mechanisms or protocols. Per RFC 6815, the tests defined herein MUST NOT be performed on production networks. The use of dedicated test IP address ranges per RFC 2544 Appendix C (198.18.0.0/15) is RECOMMENDED to prevent accidental interaction with production infrastructure. When RDMA/RoCEv2 traffic is used, the test environment SHOULD be isolated from production RDMA fabrics to prevent QP number space collisions or inadvertent PFC propagation. When UET traffic is used (Section 6), the test environment MUST ensure that UDP port 4793 traffic does not leak to production networks, and that PDC identifier spaces are isolated. UET's optional transport security sub-layer (TSS) SHOULD NOT be enabled during performance benchmarking unless transport security overhead is explicitly being measured.

15. IANA Considerations

This document makes no request of IANA.

16. References

16.1. Normative References

[RFC124] Bradner, S., "Benchmarking Terminology for Network Interconnection Devices", RFC 1242, July 1991.

[RFC2119] Bradner, S., "Key words for use in RFCs to Indicate Requirement Levels", BCP 14, RFC 2119, March 1997.

[RFC2544] Bradner, S. and J. McQuaid, "Benchmarking Methodology for Network Interconnect Devices", RFC 2544, March 1999.

[RFC2889] Mandeville, R. and J. Perser, "Benchmarking Methodology for LAN Switching Devices", RFC 2889, August 2000.

[RFC8174] Leiba, B., "Ambiguity of Uppercase vs Lowercase in RFC 2119 Key Words", BCP 14, RFC 8174, May 2017.

[RFC8238] Avramov, L. and J. Rapp, "Data Center Benchmarking Terminology", RFC 8238, August 2017.

[RFC8239] Avramov, L. and J. Rapp, "Data Center Benchmarking Methodology", RFC 8239, August 2017.

[RFC9004] Morton, A., "Updates for the Back-to-Back Frame Benchmark in RFC 2544", RFC 9004, May 2021.

[UEC-1.0] Ultra Ethernet Consortium, "Ultra Ethernet Transport (UET) Specification 1.0", June 2025, https://ultraethernet.org.

16.2. Informative References

[RFC6815] Bradner, S., et al., "Applicability Statement for RFC 2544", RFC 6815, November 2012.

[RFC7432] Sajassi, A., et al., "BGP MPLS-Based Ethernet VPN", RFC7432, February 2015.

[EVPN-BENCH] Jacob, S. and K. Tiruveedhula, "Benchmarking Methodology for EVPN and PBB-EVPN", draft-kishjac-bmwg-evpntest-10, June 2018.

[LLM-BENCH] Gaikwad, et al., "Benchmarking Methodology for Large Language Model Serving", draft-gaikwad-llm-benchmarking-methodology-00, January 2026.

[META-ROCE] Gangidi, A., et al., "RDMA over Ethernet for Distributed AI Training at Meta Scale", ACM SIGCOMM 2024.

[DCQCN] Zhu, Y., et al., "Congestion Control for Large-Scale RDMA Deployments", ACM SIGCOMM 2015.

[NCCL] NVIDIA, "NVIDIA Collective Communications Library (NCCL)", https://developer.nvidia.com/nccl.

[UEC] Ultra Ethernet Consortium, "Ultra Ethernet Transport (UET) Specification", https://ultraethernet.org.

[LIBFABRIC] OpenFabrics Interfaces Working Group, "libfabric: Open Fabric Interfaces" https://ofiwg.github.io/libfabric/.

[POLLARA] AMD, "AMD Pensando Pollara 400 SmartNIC",

https://www.amd.com/en/products/accelerators/pensando.html.

[MLPERF] MLCommons, "MLPerf Training Benchmark Suite",

Appendix A: KPI-to-Test Mapping Summary

+-------------------------+----------+-------------------------------+------------------------------+

| KPI                     | Test     | Measurement Method            | Reporting Unit               |

|                         | Section  |                               |                              |

+-------------------------+----------+-------------------------------+------------------------------+

| Throughput Rate         | 5.1      | Binary search, zero-loss      | Tbps, % line rate            |

+-------------------------+----------+-------------------------------+------------------------------+

| Latency (P99)           | 5.2      | Tagged frame,                 | microseconds                 |

|                         |          | loaded / unloaded             |                              |

+-------------------------+----------+-------------------------------+------------------------------+

| Burst Absorption        | 5.3      | Max burst without loss        | frames, bytes                |

+-------------------------+----------+-------------------------------+------------------------------+

| ECN Accuracy            | 7.1      | Queue depth vs. marking       | threshold deviation %        |

+-------------------------+----------+-------------------------------+------------------------------+

| PFC Behavior            | 7.2      | Incast sweep N=2..64          | PAUSE events/sec, duration   |

+-------------------------+----------+-------------------------------+------------------------------+

| DCQCN Convergence       | 7.3      | Rate stabilization after      | microseconds                 |

|                         |          | onset                         |                              |

+-------------------------+----------+-------------------------------+------------------------------+

| PFC Deadlock            | 7.4      | Cyclic adversarial traffic    | observed/reported,           |

|                         |          |                               | watchdog timer events        |

+-------------------------+----------+-------------------------------+------------------------------+

| ECMP Imbalance          | 8.1      | MMR, JFI per QP count         | dimensionless ratios         |

+-------------------------+----------+-------------------------------+------------------------------+

| DLB Efficacy            | 8.2      | Throughput delta vs. ECMP     | %, out-of-order rate         |

+-------------------------+----------+-------------------------------+------------------------------+

| Spray Efficacy          | 8.3      | JFI, retransmission rate      | dimensionless, retx/sec      |

+-------------------------+----------+-------------------------------+------------------------------+

| AllReduce BusBW         | 9.1      | CCL benchmark                 | Gbps per accelerator         |

+-------------------------+----------+-------------------------------+------------------------------+

| AlltoAll JCT            | 9.2      | CCL benchmark                 | seconds per iteration        |

+-------------------------+----------+-------------------------------+------------------------------+

| AllGather BusBW         | 9.3      | CCL benchmark                 | Gbps per accelerator         |

+-------------------------+----------+-------------------------------+------------------------------+

| Synthetic JCT Ratio     | 10.1     | Measured / Roofline           | dimensionless                |

+-------------------------+----------+-------------------------------+------------------------------+

| MLPerf JCT              | 10.2     | Time-to-train                 | minutes, tokens/sec          |

+-------------------------+----------+-------------------------------+------------------------------+

| Multi-Tenant Impact     | 10.3     | Contention / Baseline JCT     | interference factor          |

+-------------------------+----------+-------------------------------+------------------------------+

| Scale Limit             | 11.1     | Max N with JCT Ratio <= 1.10  | accelerator count            |

+-------------------------+----------+-------------------------------+------------------------------+

| Failover Time           | 11.2     | Loss duration on link fail    | microseconds                 |

+-------------------------+----------+-------------------------------+------------------------------+

| 24h Stability           | 12.1     | JCT Ratio std deviation       | dimensionless                |

+-------------------------+----------+-------------------------------+------------------------------+

| UET Throughput (RUD)    | 6.1      | Binary search per             | Gbps, % line rate            |

|                         |          | transport service             |                              |

+-------------------------+----------+-------------------------------+------------------------------+

| UET First-Packet        | 6.2      | PDC establish + first data    | microseconds                 |

| Latency                 |          |                               |                              |

+-------------------------+----------+-------------------------------+------------------------------+

| UET Spray Efficacy      | 6.3      | JFI/MMR under RUD spray       | dimensionless, OOO rate      |

+-------------------------+----------+-------------------------------+------------------------------+

| UET PFC-Free Loss Rate  | 6.4      | Incast without PFC enabled    | %, retx overhead             |

+-------------------------+----------+-------------------------------+------------------------------+

| LLR Retry Latency       | 6.5      | Per-hop error recovery time   | nanoseconds                  |

+-------------------------+----------+-------------------------------+------------------------------+

| Packet Trimming Savings | 6.5      | BW saved during congestion    | % bandwidth                  |

+-------------------------+----------+-------------------------------+------------------------------+

| CBFC vs PFC HOL         | 6.5      | Head-of-line blocking         | microseconds                 |

| Blocking                |          | duration                      |                              |

+-------------------------+----------+-------------------------------+------------------------------+

| UET Collective BusBW    | 6.6      | AllReduce/AlltoAll over UET   | Gbps per accelerator         |

+-------------------------+----------+-------------------------------+------------------------------+

| PDC Establishment Rate  | 6.7      | Sustained PDC creation rate   | PDCs/second                  |

+-------------------------+----------+-------------------------------+------------------------------+

| Max Concurrent PDCs     | 6.7      | Scale limit per NIC           | count                        |

+-------------------------+----------+-------------------------------+------------------------------+

Appendix B: ASIC Feature Categories for AI Fabric Benchmarking (Informational)

This appendix identifies the ASIC feature categories relevant to AI fabric performance. Implementers SHOULD document which of the following categories are present and enabled on the DUT during benchmarking.

Specific vendor names and products are intentionally omitted, this appendix guides documentation of DUT capabilities, not product comparison.

+--------------------+----------------+------------------------+--------------------------------+

| Feature            | Sub-types      | Relevance to           | What to Report                 |

| Category           |                | AI Fabric              |                                |

+--------------------+----------------+------------------------+--------------------------------+

| Aggregate          | ASIC-level     | Determines cluster     | Total Tbps; per-port           |

| Switching BW       | capacity       | scale, bisection BW    | speed (400/800GbE)             |

+--------------------+----------------+------------------------+--------------------------------+

| Buffer             | Shared, VOQ,   | Microburst absorption, | Buffer type; total bytes;      |

| Architecture       | Cut-through    | PFC behavior,          | shared vs. dedicated split;    |

|                    |                | lossless operation     | per-port/queue allocation      |

+--------------------+----------------+------------------------+--------------------------------+

| Packet             | Per-flow,      | ECMP load balancing    | Supported granularities;       |

| Distribution       | Per-packet,    | quality and            | in-fabric reorder buffer       |

|                    | Flowlet        | reordering risk        | present (yes/no)               |

+--------------------+----------------+------------------------+--------------------------------+

| Congestion         | ECN marking,   | DCQCN convergence      | ECN granularity (port/         |

| Control            | PFC, DCQCN     | and lossless           | queue/VOQ); PFC priorities;    |

|                    |                | behavior               | DCQCN parameter range          |

+--------------------+----------------+------------------------+--------------------------------+

| Adaptive           | Flowlet, ECMP, | Load balancing         | Algorithm type; flowlet        |

| Routing            | Spray,         | quality under          | gap timer range;               |

|                    | Topology-aware | collective patterns    | topology-aware support         |

+--------------------+----------------+------------------------+--------------------------------+

| Telemetry          | Per-port,      | Required for KPI       | Monitoring granularity;        |

|                    | Per-queue,     | measurement during     | streaming interval;            |

|                    | Per-flow       | benchmarking           | INT support                    |

+--------------------+----------------+------------------------+--------------------------------+

| Cluster Scale      | 2-tier,        | Applicable topology    | Max cluster size per           |

| Support            | 3-tier         | scales                 | topology; ASIC count           |

+--------------------+----------------+------------------------+--------------------------------+

Notes: All values MUST be reported based on vendor documentation or measured capability during benchmarking. The feature categories above are not exhaustive; additional DUT capabilities that affect benchmark results MUST also be documented.

Appendix C: RoCEv2 Test Frame Format

The test frame for RoCEv2 benchmarking follows this structure:

+--------+--------------------+------+------------------------------------------+

| Offset | Field              | Size | Value / Description                      |

+--------+--------------------+------+------------------------------------------+

| 00     | Ethernet Dst MAC   | 6B   | DUT next-hop MAC                         |

+--------+--------------------+------+------------------------------------------+

| 06     | Ethernet Src MAC   | 6B   | Test equipment MAC                       |

+--------+--------------------+------+------------------------------------------+

| 12     | EtherType          | 2B   | 0x8100 (802.1Q) or 0x0800 (IPv4)         |

+--------+--------------------+------+------------------------------------------+

| 14     | VLAN Tag (optional)| 4B   | PCP=3 (RoCEv2 priority), VID             |

+--------+--------------------+------+------------------------------------------+

| 18     | IPv4 Header        | 20B  | DSCP=26 (ECN-capable), Proto=17 (UDP)    |

+--------+--------------------+------+------------------------------------------+

| 38     | UDP Header         | 8B   | DstPort=4791 (RoCEv2), SrcPort=var       |

+--------+--------------------+------+------------------------------------------+

| 46     | BTH (Base          | 12B  | OpCode, DstQP, PSN, P_Key                |

|        | Transport Header)  |      |                                          |

+--------+--------------------+------+------------------------------------------+

| 58     | RETH (if Write)    | 16B  | VA, R_Key, DMA Length                    |

+--------+--------------------+------+------------------------------------------+

| 74     | Payload            | var  | Test data (incrementing octets)          |

+--------+--------------------+------+------------------------------------------+

| var    | ICRC               | 4B   | Invariant CRC                            |

+--------+--------------------+------+------------------------------------------+

| var+4  | FCS                | 4B   | Ethernet Frame Check Sequence            |

+--------+--------------------+------+------------------------------------------+

Appendix D: UET (Ultra Ethernet Transport) Frame Format

The test frame for UET benchmarking follows the UEC Specification 1.0 wire format. UET runs over UDP/IP using IANA-assigned destination port 4793. The frame structure is:

+--------+----------------------+------+------------------------------------------+

| Offset | Field                | Size | Value / Description                      |

+--------+----------------------+------+------------------------------------------+

| 00     | Ethernet Dst MAC     | 6B   | DUT next-hop MAC                         |

+--------+----------------------+------+------------------------------------------+

| 06     | Ethernet Src MAC     | 6B   | Test equipment MAC                       |

+--------+----------------------+------+------------------------------------------+

| 12     | EtherType            | 2B   | 0x8100 (802.1Q) or 0x0800 (IPv4)         |

+--------+----------------------+------+------------------------------------------+

| 14     | VLAN Tag (optional)  | 4B   | PCP=3 (UET priority class), VID          |

+--------+----------------------+------+------------------------------------------+

| 18     | IPv4 Header          | 20B  | DSCP=26, ECN=ECT(0), Proto=17 (UDP)      |

+--------+----------------------+------+------------------------------------------+

| 38     | UDP Header           | 8B   | DstPort=4793 (UET), SrcPort=entropy      |

+--------+----------------------+------+------------------------------------------+

| 46     | UET Common Header    | 16B  | Version, OpCode, PDC ID, PSN,            |

|        |                      |      | Entropy Value, Flags                     |

+--------+----------------------+------+------------------------------------------+

| 62     | SES Header           | var  | Operation-specific (Write/Send/etc.)     |

|        | (Semantic)           |      |                                          |

+--------+----------------------+------+------------------------------------------+

| var    | PDS Header           | var  | Sequence, Credit, Ack fields             |

|        | (Pkt Delivery)       |      |                                          |

+--------+----------------------+------+------------------------------------------+

| var    | CMS Header           | var  | ECN feedback, rate signals               |

|        | (Cong. Mgmt)         |      |                                          |

+--------+----------------------+------+------------------------------------------+

| var    | Payload              | var  | Application data                         |

+--------+----------------------+------+------------------------------------------+

| var    | ICRC                 | 4B   | Invariant CRC                            |

+--------+----------------------+------+------------------------------------------+

| var+4  | FCS                  | 4B   | Ethernet Frame Check Sequence            |

+--------+----------------------+------+------------------------------------------+

Key differences from RoCEv2 (Appendix C):

1. UDP Destination Port: UET uses port 4793 (IANA-assigned) vs. RoCEv2 port 4791.

2. Entropy Value: The UET common header includes an explicit entropy field for ECMP path selection, replacing RoCEv2 's reliance on UDP source port variation. The test equipment MUST vary this field to achieve uniform path distribution.

3. Transport Service Indicator: The UET header encodes the transport service (ROD, RUD, RUDI, UUD) in the OpCode/Flags fields. Tests MUST set this field to match the service being benchmarked.

4 . PDC Identifier: The connectionless PDC ID field replaces RoCEv2 's Destination QP. PDC IDs are ephemeral and may be reused; the test equipment MUST track PDC lifecycle for accurate measurement.

5 . Layered Sub-Headers: UET uses four sub-layers (SES, PDS, CMS, TSS) with variable-length headers. The exact header layout depends on the transport service and operation. Implementations MUST follow [UEC-1.0]

Section 4 for wire format details.

6. Optional Link Layer Headers: When LLR, Packet Trimming, or PRI features are enabled, additional link-layer framing may be present between the Ethernet header and the IP header. The test equipment MUST be configured to recognize and correctly parse these optional headers.

+------------------+------------------+------------------+-------------------------------+

| Field            | RoCEv2 Value     | UET Value        | Notes                         |

+------------------+------------------+------------------+-------------------------------+

| UDP Dst Port     | 4791             | 4793             | IANA-assigned for each        |

|                  |                  |                  | protocol                      |

+------------------+------------------+------------------+-------------------------------+

| Transport        | QP Number        | PDC ID           | Connectionless in UET         |

| Endpoint         | (24b)            | (variable)       |                               |

+------------------+------------------+------------------+-------------------------------+

| Sequence Number  | PSN (24b)        | PSN (extended)   | Larger range in UET for       |

|                  |                  |                  | RUD OOO tolerance             |

+------------------+------------------+------------------+-------------------------------+

| Congestion       | ECN bits only    | ECN + CMS        | Sender + receiver             |

| Signal           |                  | sub-header       | signals in UET                |

+------------------+------------------+------------------+-------------------------------+

| Entropy Source   | UDP src port     | Explicit entropy | Deterministic spray in UET    |

|                  |                  | field            |                               |

+------------------+------------------+------------------+-------------------------------+

| Ordering         | Always in-order  | Per-service      | RUD allows OOO delivery       |

| Guarantee        | (RC)             | (ROD/RUD)        |                               |

+------------------+------------------+------------------+-------------------------------+

| Min Header       | ~74B (Write)     | ~78B (est.       | Slight increase for           |

| Overhead         |                  | Write)           | sub-layer headers             |

+------------------+------------------+------------------+-------------------------------+

Authors' Addresses

Fernando Calabria
Cisco
Email: fcalabri@cisco.com

Carlos Pignataro
Blue Fern Consulting
Email: carlos@bluefern.consulting

Contributions and review are solicited from the BMWG mailing list (bmwg@ietf.org) and the broader AI networking community. The BMWG chairs and Area Director are identified at https://datatracker.ietf.org/group/bmwg/about/.